Introduction of 520 DC gear motor

- Product Introduction

520 Hall coding DC gear motor is a high torque, high load motor, 12V DC motor. Hall speed measuring code disk is a speed measuring module using Hall sensor encoder. It is equipped with strong magnetic code disk and outputs pulse signals through AB two-phase, which can detect the rotation direction and speed of the motor and is suitable for intelligent cars with high speed requirements. Such as McNamee wheel car, balance car, navigation and positioning car.

- Parameter Description

  - Specifications

| Parameters | JGB37-520R20-12 | JGB37-520R30-12 | JGB37-520R60-12 | JGB37-520R90-12 |
| --- | --- | --- | --- | --- |
| Reduction ratio | 1:20 | 1:30 | 1:60 | 1:90 |
| No-load speed | 500rpm | 320rpm | 170rpm | 110rpm |
| Rated speed | 390rpm | 280rpm | 146rpm | 85rpm |
| Stall-rotor torque | 3.7kg*cm | 5.8kg*cm | 9.2kg*cm | 15kg*cm |
| Rated torque | 1kg*cm | 1.2kg*cm | 1.9kg*cm | 2.6kg*cm |

| Stall current | 3.9A | 3.2A | 3.2A | 3.2A |
| --- | --- | --- | --- | --- |
| Rated current | 0.36A | 0.36A | 0.53A | 0.36A |
| Rated power | Approximately 9W | Approximately 9W | About 7W | About 83W |
| Rated voltage | 12V |  |  |  |
| Encoder Class Type | AB two-phase encoder |  |  |  |
| Motor type | permanent magnet brush |  |  |  |
| Output shaft | Diameter 6mmD type eccentric shaft |  |  |  |
| Encoder electric Pressure | 3.3~5V |  |  |  |
| Number of magnetic loops | 11 line. |  |  |  |
| Interface Type | PH2.0-6PIN |  |  |  |
| Function | With pull-up shaping, the microcontroller can directly read the signal pulse |  |  |  |
| Single motor weight Quantity | 152+1G |  |  |  |

  - Under the condition that the internal motor of the geared motor remains unchanged, the relationship between the gear reduction ratio in the reduction box and the speed and torque: the larger the reduction ratio, the greater the torque, and the smaller the speed.

  - Motor model description:

  - Pin Description

| Label | Description |
| --- | --- |
| GND | Hall power supply negative pole |
| C1 | Hall C1 output signal, square wave |
| C2 | Hall C2 output signal, square wave |
| VCC | Hall power supply positive pole |
| M1 | Positive pole of motor power supply |
| M2 | Negative pole of motor power supply |

Remarks:

1. The voltage between the VCC-GND is determined according to the power supply voltage of the single-chip microcomputer used, and 3.3V or 5V is generally used.

2. The frequency of the output square wave of C1 and C2 is related to the motor speed and the number of poles of the magnetic sheet used.

3. The voltage between M1 and M2 depends on the motor voltage used.